NO111419A_Camui_305A/306A_CalBryte630AM_10Ca_2Ca

cells are~ 90% confluency,   1DAT,

Laser Calibration 930 nm (1850mW at the laser screen, software) not calibrated

7%=0.8mW

8%=0.97mW

9%=1.1mw

10%=1.2mW

11%=1.32mW

12%=1.45mW

13%=1.6mW

14%=1.7mW

15%=1.86mW

Laser Calibration 1010 nm (920mW at the laser screen, software)-

(The values inside the parenthesis are as of November 12th, 2109)

9% = 1.09mW  (1.2)

10% = 1.2mW

12% = 1.5mW

13% = 1.6mW

14% = 1.7mW

15% = 1.9mW

16% = 2.0mW

17% = 2.2 mW

18% = 2.3 mW

Laser 1010  17 %

HBSS 0Ca,

1uL aliquots of 8 mM conc were prepared by Nick on Nov 14th, by adding 20ul of DMSO to 50uL dye. Then 1ul of this dye stock was added to 500ul of (DMEM+FBS) /0.02% pluronic PF-127.  The latter was prepared  by adding 3.5 ul of 10%pluronic (in DW prepared 11.14.19) to 10 ml of DMEM.

get a final dye conc of about 4uM. 2 mM and 10 mM solutions contained 1uMlatrunculin A and 50nM Caliculin A (Caliculin prepared from old stock and Latrunculin prepared from stock 11.14.19)

128x128; Aver =20, 10x1um stack

Loc1- 3m, 1b lt 1.81 (15)

Loc2- 2m

Loc3- 6m lt 1.78

Loc4- 3m

Loc 5-2m

Loc6- 3m lt 1.79

Loc7- 3m, 3b lt 1.8

Loc8- 5m, 2b lt 1.8

Loc 9-4m, 3b lt 1.79

Loc10-4m, 1b lt 1.77

Loc 10 R lt 1.8

Loc 10 R lt 1.77

Loc11- 2m, 2b

Loc 12-2m (good)

Loc13- 2m, 2b

Loc14-

Loc15-

Loc16-

Loc17-

Loc18-

Loc19-

Loc20 -

Loc21 –

Loc22-

Loc23-

Loc24-

Loc25-

Loc26-

Loc27

B

AI  pos 20 (position – 12- dendritic imaging)

10 im stack  20 aver (77 sec total)

Start with HBSS/0Ca (25mMHEPES) (prepared from HBSS/0Ca (Nick 11.08.19)

After img5 start HBSS/10Calcium 100uM ATP

After 12 Start with HBSS/2Ca

After 19, start recirculating HBSS/2Ca

Img 39 end